Week 1 Report: Foundations of Design Engineering

Smart Temperature-Controlled Electronic Enclosure

Prepared by: Nimrah Saleem | Department of Electronic Engineering, NED University

# 1. Introduction

Design engineering is the process of converting a functional requirement into a practical product or system, considering performance, dimensions, materials, manufacturability, and safety. CAD (Computer-Aided Design) is used to create accurate 2D drawings and 3D models, design calculations justify dimensions and component choices, and engineering drawings communicate the information needed to manufacture the product.

# 2. Problem Statement

Electronic circuits generate heat during operation, and excessive temperature can reduce reliability or cause malfunction. A compact enclosure must provide physical protection while allowing heat to escape. The challenge is to design a small enclosure with organized component mounting, ventilation, cable access, and an automatic cooling mechanism.

# 3. Proposed System

The system uses a temperature sensor connected to an Arduino controller. The controller compares the measured temperature with predefined thresholds and controls a cooling fan and status indicators (LEDs and buzzer) accordingly.

System Flow: LM35 Temperature Sensor → Arduino UNO → Fan / LEDs / Buzzer

Below 30°C — Fan OFF, Green LED (normal operation)

30–39°C — Fan ON, Yellow LED (active cooling)

≥40°C — Fan ON, Red LED + Buzzer (alert)

# 4. CAD Design of the Enclosure

The enclosure was modeled with outer dimensions of 150 mm (L) × 100 mm (W) × 60 mm (H) and a 3 mm wall thickness, including mounting standoffs, a fan opening, ventilation slots, and panel openings for LEDs, buzzer, and cables.

Figure 1: 3D CAD Model of the Enclosure (Closed)

Figure 2: Internal Assembly View — PCB and Component Placement

# 5. Design Calculations

Enclosure volume: V = 150 × 100 × 60 = 900,000 mm³ = 0.9 L

Fan power: P = 5V × 0.20A = 1.0 W

Controller/sensor power: P = 5V × 0.10A = 0.50 W

Total estimated load with 25% margin: 1.5 W × 1.25 = 1.875 W

Wall thickness: 3 mm (prototype value); Mounting: M3 fasteners

# 6. Engineering Drawings

The drawing package includes front, top, side, and rear views with dimensions in millimeters, plus an isometric and exploded view for assembly clarity.

Figure 3: Engineering Drawing — Orthographic Views (mm) and Exploded Assembly

# 7. Conclusion

Week 1 covered the fundamentals of the design engineering workflow — CAD modeling, design calculations, and engineering drawings — applied here to a temperature-controlled electronic enclosure. The CAD environment was set up and used to produce a dimensioned 3D model and drawing package, forming the foundation for prototype development in the following weeks.